Elena Chong

Mobile Robotics Independent Study

Winter 2016

Line Maze Solver Final Report

INTRODUCTION

The Micromouse Maze Robot competition served as an inspiration for this independent study. The idea of programming a mobile robot capable of remembering the route it took, deciding which one is right and which one is a dead-end, then extracting the fastest route from the starting point to the final point was something worth exploring. To scale this project down, I decided to explore cognitive robotics by building and programming a robot capable of solving a simple line maze. The goal of this independent study was to autonomously get the robot from the Start to the Finish point.

A line maze has black line that goes from the start point to the finish point. In order to make it a maze, random lines are added to test the decision-making logic of the robot. An example of a line maze can be found in the Appendix. The ultimate goal of the robot is to follow the lines and find its way from Start to Finish without making any wrong turns. To accomplish this, the robot is first allowed to go through an exploration phase, where it can freely take whichever route it wants and records its path in the memory. If it has taken a turn that leads to a dead-end, it will remember such mistake and fix it by replacing the wrong path with a correct one.

In addition to the line maze solver project, it was a requirement to read articles and lecture notes from the ECE425: Mobile Robotics course. Quizzes were assigned twice a week for the first seven weeks. During the course of this independent study, I worked with Amanda McLean. Each of us with our own robot.

MAZE LINE SOLVER THEORY

To solve the line maze, we need to have a systematic procedure to test the logic for machine learning.

Left-Hand Rule:

Take a left over straight

- Take a straight over right

- Take a right

- U-turn for dead-end

Right-Hand Rule:

- Take a right over straight

- Take a straight over left

- Take a left

- U-turn for dead-end

If we follow one of those rules, we should be able to get out of a line maze with no loop. In my implementation, I used the right-hand rule.

The method of solving the maze with making no turning mistakes can be accomplished by keeping track of turns made. In order to do that, the robot needs to go through the maze twice. Every time we make a turn of interest, we record that with a character letter. When either (L)eft, (R)ight, or S(traight) are in between a (U)Turn for dead-end, we make a substitution, for example:

| TURN COMBINATION LEFT-HAND RULE | TURN COMBINATION RIGHT-HAND RULE | SUBSTITUTION |
| --- | --- | --- |
| LUR | RUL | B |
| LUS | RUS | R |
| LUL | RUR | S |
| SUL | SUR | R |
| SUS | SUS | B |
| RUL | LUR | B |

The U-Turns are going to tell the robot that it has made a wrong turn. The final stage of solving the maze should not go through any dead-ends.

How the substitution works?

Let’s take for example the following maze.

Make a U-Turn

Turn right over straight

The robot will start from point A. Using the RHR, the robot will first make a Right Turn, so we record that to our memory (array) as a R.

Memory: R

Now, the robot will reach the red intersection and make a right again.

Memory: RR

The robot will encounter a dead-end, so it will make a U-turn.

Memory: RRU

The robot will then make a right turn again.

Memory: RRUR

We see that RUR is one of the combinations, and it is replaced by a straight, which is the correct path if the robot didn’t make the right turn in the red intersection in the first place.

Memory: RS

Then, the robot will make a right, and another right to reach the final destination point B.

Memory: RSRR

After going through the maze once and recorded all the mistakes, the robot will just perform the path from its memory, which is RSRR. It should take the robot from point A to point B with no mistaken turns.

Testing theory:

Mazes designed for testing purposes:

Maze 1. Line following, no need for decision-making

Maze 1 Goals:

- Follow the lines to get from circle to rectangle. Accomplished

- Follow the lines to get from rectangle to circle. Accomplished

Maze 2. Simple decision-making test – No dead-end

Maze 2 Goals:

- Follow the lines with Right-Hand Rule – shouldn’t turn left at any point. Accomplished

- Pass the T intersection, make a right. Accomplished

Maze 3. Take a right over straight to get to finish point B in cross (+) intersection

Maze 3 Goals:

- Take a right at the cross-intersection to get to point B. Accomplished

Make a U-Turn

Turn right over straight

Maze 4. Take a right to get to a dead-end

Maze 4 Goals:

- Follow the Right-Hand Rule

- Take a right over going straight

- Make a U-Turn on the spot

- Take a right to go to finish point B without making any left turns

5

3

4

2

1

Make a U-Turn

Make a U-Turn

Make a U-Turn

Turn right over straight

Maze 5. Complex Decision Making

Maze 5 Goals:

- Make a right over straight to get to the cross-intersection

- Make a right, arrive at a dead-end

- Escape the dead-end

- Make a right, fix the dead-end

- Make another right, fix the dead-end

- Make another right, and a (5) right to go to point B

Maze 6. Line maze used for testing the overall logic.

Maze 6 Goals:

- Go through the entire maze doing RHR decisions.

- Solve the dead-ends

- Get to the finish point

- Memorize the mistakes and correct them

Finite State Machine for further testing

After testing code for several weeks and not getting through the end of the maze, we decided to finally try to tackle the maze line solver algorithm by using a finite state machine since there are many cases that the robot needs to take into account. We keep track of the previous state and the next state, the latter defines the motion of the robot.

The following table shows the previous state, sensors input, and next state:

| Previous State | input | Next State |
| --- | --- | --- |
| start | 000 | start |
| straight | 000 | u-turn |
| u-turn | 000 | u-turn |
| decide left | 000 | left |
| decide right | 000 | right |
| start | 010 | straight |
| u-turn | 010 | straight |
| right | 010 | straight |
| decide left | 010 | straight |
| left | 010 | straight |
| decide right | 010 | right |
| straight | 011 | right |
| straight | 110 | decide left |
| straight | 111 | decide right |
| decide right | 111 | finished |
| finished | 111 | finished |
| finished | 010 | straight |

Table 1. Finite State Machine for three line following sensors

Figure 1. Draft of our Finite State Machine Diagram

HARDWARE

For the first four weeks, we researched the possible hardware that we could use to build our robots. We explored the QuickBot MOOC and analyzed the different microcontroller that we could use for the connections and programming interfaces. We thought about using the Beagle Bone Black for the QuickBot, and got two BBB from the Parts Room. We realized later that the Magician Chassis were discontinued by Sparkfun, so we looked at other options. We noticed that students were using the SparkFun RedBot for the CSSE120 course. The SparkFun RedBot is the updated bot of the Magician Chassis, so we decided to go with the RedBot Basic Kit. We assembled the kit following the instruction given on the SparkFun website during Week 5. The kit comes with the RedBot Mainboard, which comes pre-programmed with the UNO bootloader, shadow chassis, accelerometer, 3x line follower sensors, 2x gearmotor, and 2x wheels (65mm). We noticed that we needed the wheel encoders, so we purchased them separately and installed them afterwards.

Figure 2. Assembled red-bot basic kit

SOFTWARE

The RedBot mainboard has a ONE bootloader on it, so it is necessary to use the Arduino IDE for development. I downloaded an Arduino extension for Microsoft Visual Studio Community 2015 using Visual Micro. This allows me to code in Arduino using MVS and upload to the board just like the Arduino IDE.

I started developing my own maze line solver algorithm on top of the line following example from the RedBot library, which follows black lines with no decision-making. In order to install the RedBot library, go to the SparkFun RedBot webpage and download the library. Then, place the library in the following directory: C:\Users\<Your_User_Name>\Documents\Arduino\libraries.

The library provides useful functions for controlling the robot. Because of how the motor drive function for left and right were programmed inside the RedBot library, we should note the following for the Redbot motion:

| RedBot Motion |  |  |
| --- | --- | --- |
| Motion | Left Speed | Right Speed |
| Forward | - | + |
| Reverse | + | - |
| CCW | + | + |
| CW | - | - |

Table 2. Sign chart for controlling left and right motors of the RedBot

The source codes for the testing of each maze line solver are shown in the Appendix. The solution for Maze 1 is the example code provided from SparkFun RedBot line following tutorial.

In the source code for Maze 2 and Maze 3, I added the following:

if ((leftSensors() > THRESHOLD) && (centerSensors() > THRESHOLD) && (rightSensors() > THRESHOLD))

{

while ((leftSensors() > THRESHOLD) && (centerSensors() > THRESHOLD) && (rightSensors() > THRESHOLD)) {

Serial.println("looping");

leftSpeed = -SPEED - 20;

rightSpeed = -SPEED ;

motors.leftMotor(leftSpeed);

motors.rightMotor(rightSpeed);

}

}

This code gives some extra logic to the robot. If the three line sensors are registering black, then turns right. Right after that, the line following code kicks in to make sure the robot keeps following the line.

The code for making a U-turn is the following:

#define USPEED 110

#define UDELAY 1000

leftSpeed = -USPEED;

rightSpeed = -USPEED;

motors.leftMotor(leftSpeed);

motors.rightMotor(rightSpeed);

delay(UDELAY);

motors.brake();

This code makes the robot spin 180 degree (U-turn). The problem that I am facing right now it is that the sensors aren’t useful enough to tell if a U-turn is actually needed. It is getting confused because sometimes all three sensors could be below the threshold.

CONCLUSION

These 10 weeks, I was able to learn more about mobile robots. The need for sensor redundancy like having more IR sensors for maze solving, and wheel encoders for gathering more data and controlling the drive of the wheels. After five weeks of trying to come up with a good programming solution to solve the line maze, I came to realize that the robot were having a lot of trouble with the intersections. Since the bot has only three line follower sensors, it was challenging to map the correct intersection. It is recommended to use at least 5 line sensors for better identification of intersections and logic performances. I was not able to solve the maze because of the U-Turn issue. However, I hope to have that implemented and working in the future.

Future Work

It’s still possible to solve the maze using three sensors. I still need it to make a U-Turn when needed. After that, I will need to code the actual cognition of the robot; been able to store wrong routes and fix them so that it can get through the maze with no wrong turns.

APPENDIX

Code for Maze Line Solver 1 :

Modified from the line following example from SparkFun Redbot

#include <RedBot.h>

RedBotSensor left = RedBotSensor(A3);   // initialize a left sensor object on A3

RedBotSensor center = RedBotSensor(A6); // initialize a center sensor object on A6

RedBotSensor right = RedBotSensor(A7);  // initialize a right sensor object on A7

#define THRESHOLD 800

#define SPEED 60  // sets the nominal speed. Set to any number from 0 - 255.

RedBotEncoder encoder = RedBotEncoder(A2, 10);  // initializes encoder on pins A2 and 10

RedBotMotors motors;

int leftSpeed;   // variable used to store the leftMotor speed

int rightSpeed;  // variable used to store the rightMotor speed

void setup()

{

Serial.begin(9600);

delay(2000);

Serial.println("IR Sensor Readings: ");

delay(500);

}

void loop()

{

Serial.print(left.read());

Serial.print("\t");  // tab character

Serial.print(center.read());

Serial.print("\t");  // tab character

Serial.print(right.read());

Serial.println();

// if on the line drive left and right at the same speed (left is CCW / right is CW)

if (center.read() > THRESHOLD)

{

leftSpeed = -SPEED;

rightSpeed = SPEED;

}

// if the line is under the right sensor, adjust relative speeds to turn to the right

if (right.read() > THRESHOLD)

{

leftSpeed = -(SPEED + 50);

rightSpeed = SPEED - 50;

//rightSpeed = -SPEED + 10;

}

// if the line is under the left sensor, adjust relative speeds to turn to the left

if (left.read() > THRESHOLD)

{

leftSpeed = -(SPEED - 50);

rightSpeed = SPEED + 50;

}

// if all sensors are on black or up in the air, stop the motors.

// otherwise, run motors given the control speeds above.

if ((left.read() > THRESHOLD) && (center.read() > THRESHOLD) && (right.read() > THRESHOLD))

{

motors.stop();

}

else

{

motors.leftMotor(leftSpeed);

motors.rightMotor(rightSpeed);

}

delay(0);  // add a delay to decrease sensitivity.

}

Code for Maze Line Solver 2 :

#include <RedBot.h>

RedBotSensor left = RedBotSensor(A3);   // initialize a left sensor object on A3

RedBotSensor center = RedBotSensor(A6); // initialize a center sensor object on A6

RedBotSensor right = RedBotSensor(A7);  // initialize a right sensor object on A7

#define THRESHOLD 700

#define SPEED 60  // sets the nominal speed. Set to any number from 0 - 255.

#define USPEED 110

#define UDELAY 1000

RedBotEncoder encoder = RedBotEncoder(A2, 10);  // initializes encoder on pins A2 and 10

RedBotMotors motors;

int leftSpeed;   // variable used to store the leftMotor speed

int rightSpeed;  // variable used to store the rightMotor speed

int countsPerRev = 192;   // 4 pairs of N-S x 48:1 gearbox = 192 ticks per wheel rev

float wheelDiam = 2.56;  // diam = 65mm / 25.4 mm/in

float wheelCirc = PI*wheelDiam;  // Redbot wheel circumference = pi*D

int lCount; // variables used to store the left and right encoder counts.

int rCount;

int count =0;

int leftAvg;

int rightAvg;

int centerAvg;

boolean uReverse = false;

void setup()

{

Serial.begin(9600);

delay(2000);

Serial.println("IR Sensor Readings: ");

delay(500);

}

void loop()

{

Serial.print(left.read());

Serial.print("\t");  // tab character

Serial.print(center.read());

Serial.print("\t");  // tab character

Serial.print(right.read());

Serial.println();

// if on the line drive left and right at the same speed (left is CCW / right is CW)

if (centerSensors() > THRESHOLD)

{

leftSpeed = -SPEED;

rightSpeed = SPEED;

}

// if the line is under the right sensor, adjust relative speeds to turn to the right

if (rightSensors() > THRESHOLD)

{

leftSpeed = -(SPEED + 50);

rightSpeed = SPEED - 50;

}

// if the line is under the left sensor, adjust relative speeds to turn to the left

if (leftSensors() > THRESHOLD)

{

leftSpeed = -(SPEED - 50);

rightSpeed = SPEED + 50;

}

// if all sensors are on black or up in the air, stop the motors.

// otherwise, run motors given the control speeds above.

if ((leftSensors() > THRESHOLD) && (centerSensors() > THRESHOLD) && (rightSensors() > THRESHOLD))

{

while ((leftSensors() > THRESHOLD) && (centerSensors() > THRESHOLD) && (rightSensors() > THRESHOLD)) {

Serial.println("looping");

leftSpeed = -SPEED - 20;

rightSpeed = -SPEED ;

motors.leftMotor(leftSpeed);

motors.rightMotor(rightSpeed);

}

}

else {

motors.leftMotor(leftSpeed);

motors.rightMotor(rightSpeed);

}

Finite State Machine Code:

void updateStates() {

// input: 000

if ((left.read() < THRESHOLD) && (center.read() < THRESHOLD) && (right.read() < THRESHOLD)) {

prevState = nextState; //updating previous state to be what the previous action was.

if (prevState == start) {

nextState = start;

}

else if (prevState == straight) {

nextState = uTurn;

}

else if (prevState == uTurn) {

nextState = uTurn;

}

else if (prevState == decideLeft) {

nextState = turnLeft;

}

else if (prevState == decideRight) {

nextState = turnRight;

}

}

// input: 010

else if ((left.read() < THRESHOLD) && (center.read() > THRESHOLD) && (right.read() < THRESHOLD)) {

prevState = nextState;

if (prevState == start) {

nextState = straight;

}

else if (prevState == uTurn) {

nextState = straight;

}

else if (prevState == turnRight) {

nextState = straight;

}

else if (prevState == decideLeft) {

nextState = straight;

}

else if (prevState == turnLeft) {

nextState = straight;

}

else if (prevState == decideRight) {

nextState = turnRight;

}

else if (prevState == finished) {

nextState = straight;

}

}

// Input: 011

else if ((left.read() < THRESHOLD) && (center.read() > THRESHOLD) && (right.read() > THRESHOLD)) {

prevState = nextState;

if (prevState == straight) {

nextState = turnRight;

}

}

// Input: 110

else if ((left.read() > THRESHOLD) && (center.read() > THRESHOLD) && (right.read() < THRESHOLD)) {

prevState = nextState;

if (prevState == straight) {

nextState = decideLeft;

}

}

// Input: 111

else if ((left.read() > THRESHOLD) && (center.read() > THRESHOLD) && (right.read() > THRESHOLD)) {

prevState = nextState;

if (prevState == straight) {

nextState = decideRight;

}

else if (prevState == decideRight) {

nextState = finished;

}

else if (prevState == finished) {

nextState = finished;

}

}

// printing the values into the serial

delay(100);

Serial.print("Previous State: ");

Serial.println(prevState);

Serial.print("Current State: ");

Serial.println(nextState);

}

Maze Solver Map:

#include <RedBot.h>

RedBotSensor left = RedBotSensor(A3);   // initialize a left sensor object on A3

RedBotSensor center = RedBotSensor(A6); // initialize a center sensor object on A6

RedBotSensor right = RedBotSensor(A7);  // initialize a right sensor object on A7

#define THRESHOLD 700

#define SPEED 60  // sets the nominal speed. Set to any number from 0 - 255.

#define USPEED 110

#define UDELAY 1000

RedBotEncoder encoder = RedBotEncoder(A2, 10);  // initializes encoder on pins A2 and 10

RedBotMotors motors;

int leftSpeed;   // variable used to store the leftMotor speed

int rightSpeed;  // variable used to store the rightMotor speed

int countsPerRev = 192;   // 4 pairs of N-S x 48:1 gearbox = 192 ticks per wheel rev

float wheelDiam = 2.56;  // diam = 65mm / 25.4 mm/in

float wheelCirc = PI*wheelDiam;  // Redbot wheel circumference = pi*D

int lCount; // variables used to store the left and right encoder counts.

int rCount;

int count =0;

int leftAvg;

int rightAvg;

int centerAvg;

boolean uReverse = false;

enum state { // States

start,        // 0

straight,     // 1

turnLeft,     // 2

turnRight,    // 3

uTurn,        // 4

decideLeft,   // 5

decideRight,  // 6

finished      // 7

};

state nextState = start;

state prevState = start;

void setup()

{

Serial.begin(9600);

delay(2000);

Serial.println("IR Sensor Readings: ");

delay(500);

}

void loop()

{

Serial.print(left.read());

Serial.print("\t");  // tab character

Serial.print(center.read());

Serial.print("\t");  // tab character

Serial.print(right.read());

Serial.println();

// if on the line drive left and right at the same speed (left is CCW / right is CW)

if (centerSensors() > THRESHOLD)

{

leftSpeed = -SPEED;

rightSpeed = SPEED;

}

// if the line is under the right sensor, adjust relative speeds to turn to the right

if (rightSensors() > THRESHOLD)

{

leftSpeed = -(SPEED + 50);

rightSpeed = SPEED - 50;

}

// if the line is under the left sensor, adjust relative speeds to turn to the left

if (leftSensors() > THRESHOLD)

{

leftSpeed = -(SPEED - 50);

rightSpeed = SPEED + 50;

}

// if finish point, then stop

if ((leftSensors() > THRESHOLD) && (centerSensors() > THRESHOLD) && (rightSensors() > THRESHOLD))

{

while ((leftSensors() > THRESHOLD) && (centerSensors() > THRESHOLD) && (rightSensors() > THRESHOLD)) {

Serial.println("looping");

leftSpeed = -SPEED - 20;

rightSpeed = -SPEED;

motors.leftMotor(leftSpeed);

motors.rightMotor(rightSpeed);

}

}

// if dead-end, then U-Turn

// STILL NOT WORKING PROPERLY

if ((leftSensors() < THRESHOLD) && (centerSensors() < THRESHOLD) && (rightSensors() < THRESHOLD)) {

driveDistance(1, -SPEED, SPEED);

if ((leftSensors() < THRESHOLD) && (centerSensors() < THRESHOLD) && (rightSensors() < THRESHOLD)) {

leftSpeed = -USPEED;

rightSpeed = -USPEED;

motors.leftMotor(leftSpeed);

motors.rightMotor(rightSpeed);

delay(UDELAY);

motors.brake();

}

}

else {

motors.leftMotor(leftSpeed);

motors.rightMotor(rightSpeed);

}

delay(0);  // add a delay to decrease sensitivity.

}

int centerSensors() {

int centerS;

for (int i = 0; i < 5; i++) {

centerS = centerS + center.read();

}

return centerAvg = centerS / 5;

}

int rightSensors() {

int rightS;

for (int i = 0; i < 5; i++) {

rightS = rightS + right.read();

}

return rightAvg = rightS / 5;

}

int leftSensors() {

int leftS;

for (int i = 0; i < 5; i++) {

leftS = leftS + left.read();

}

return leftAvg = leftS / 5;

}

/********************************************

* Go forward an inch to decide next movement.

*/

void goForwardDecide() {

// move forward and decide based on values read.

Serial.println("Move forward an inch");

driveDistance(1, -SPEED, SPEED); //drive forward an inch

delay(200); //delay

if ((right.read() < THRESHOLD) && (center.read() < THRESHOLD) && (left.read() < THRESHOLD)) {

//right turn

leftSpeed = -(SPEED + 20);

rightSpeed = SPEED - 20;

}

else if ((right.read() < THRESHOLD) && (center.read() > THRESHOLD) && (left.read() < THRESHOLD)) {

leftSpeed = -SPEED;

rightSpeed = SPEED;

}

//else {

//	leftSpeed = 0;

//	rightSpeed = 0;

//}

delay(200);

driveDistance(1, SPEED, -SPEED);

delay(200);

motors.leftMotor(leftSpeed);

motors.rightMotor(rightSpeed);

delay(1000);

motors.brake();

}

/**

* Drive distance function from RedBot library examples

* distance in inches

* leftPower must be negative to drive forward

* rightPower must be positive to drive forward

*/

void driveDistance(float distance, int leftPower, int rightPower)

{

long lCount = 0;

long rCount = 0;

float numRev;

// debug

Serial.print("driveDistance() ");

Serial.print(distance);

Serial.print(" inches at ");

Serial.print(leftPower);

Serial.println(" left power.");

Serial.print(rightPower);

Serial.println(" right power.");

numRev = (float)distance / wheelCirc;

Serial.println(numRev, 3);

encoder.clearEnc(BOTH);  // clear the encoder count

motors.leftMotor(leftPower);

motors.rightMotor(rightPower);

while (abs(lCount) < numRev*countsPerRev)

{

// while the left encoder is less than the target count -- debug print

// the encoder values and wait -- this is a holding loop.

lCount = encoder.getTicks(LEFT);

rCount = encoder.getTicks(RIGHT);

Serial.print(lCount);

Serial.print("\t");

Serial.print(rCount);

Serial.print("\t");

Serial.println(numRev*countsPerRev);

}

// now apply "brakes" to stop the motors.

motors.brake();

}

REFERENCES

- Micromouse Maze: http://robogames.net/rules/maze.php

- Micromouse – Maze Solver – Theory: http://www.societyofrobots.com/member_tutorials/book/export/html/94

- Design a line maze solving robot: https://www.pololu.com/file/0J195/line-maze-algorithm.pdf

- Line Maze Solving Robot: https://www.youtube.com/watch?v=mJV-KDqHgDQ

- RedBot basic kit: https://www.sparkfun.com/products/13166

- Wheel Encoder kit: https://www.sparkfun.com/products/12629

- RedBot Library: https://github.com/sparkfun/RedBot

- RedBot assembly guide: https://learn.sparkfun.com/tutorials/assembly-guide-for-redbot-with-shadow-chassis?_ga=1.38133733.731296200.1441188245

- Robotic Maze Solving: http://patrickmccabemakes.com/tutorials/Maze_Solving/

- Maze Solver: http://letsmakerobots.com/node/33296

- http://arcbotics.com/lessons/maze-solving-home-lessons/